441/2

HOME SCIENCE

(CLOTHING CONSTRUCTION PRACTICALS)

PAPER 2

JULY/AUGUST 2024

TIME :  21/2     HOURS

MOKASA 2 E X A M I N A T I O N S

K.C.S.E TRIAL EXAMINATION

441/2

HOME SCIENCE

CLOTHING AND CONSTRUCTION

PAPER 2

MARKING SCHEME

Candidates Name: …………………………………………………………………………..

Index No.: ………………………………………………………………………………………..

Subject Teacher’s Name: …………………………………………………………………

| 1. | PRESENTATION | MAX. SCORE | ACTUAL SCORE | REMARKS |
| --- | --- | --- | --- | --- |
|  | - Work well pressed and folded - Label clearly written name, Adm, Class. - Label stitched firmly on a single fabric without concealing details - Unnecessary hanging threads, tacking and pins removed - Made up the LEFT half of the shirt | 1 11/2 1 11/2 ½ |  |  |
|  | Sub Total | 51/2 |  |  |
| 2. | CUTTING OUT AND GRAINING |  |  |  |
|  | - All eleven pieces cut out - Smooth cutting of the front bodice and on straight grain at CF - Smooth cutting of the front yoke and on straight grain at CF - Smooth cutting of the back bodice and on straight grain at CB - Smooth cutting of the back yoke and on straight grain at CB - Smooth cutting of the collar, under collar and interfacing and on straight grain at CF | 51/2 2 2 2 2 6 |  |  |
|  | Sub Total | 191/2 |  |  |
| 3 | WORKING OF THE WIDE TUCKS ON THE FRONT BODICE |  |  |  |
|  | - All four tucks made - The tucks folded at the fold line and stitched with Straight & firm stitchery - Stitching done on a double fabric - Correct width of tucks 0.5 cm to within - Tucks pressed to the side seam - Evenness of the tucks | 2 2 2 2 1 1 |  |  |
|  | Sub total | 10 |  |  |
| 4 | JOINING THE FRONT BODICE TO THE FRONT YOKE USING OVERLAID SEAM (if not give zero) |  |  |  |
|  | - The front yoke folded to W.S on the stitching line. - The front yoke placed on the R.S. of the front bodice at the stitching line. - Seam done with firm and straight stitches and close to the fold - Seam turnings trimmed to 1 cm - Seam turnings neatened together. - Appropriate neatening stitches (If not give zero from f – i) - Evenness and neatness of stitches - Good tension - Fastened on and off - Pressed and flat at the armhole and hem | ½ 1/2 1 ½ 1/2 1/2 1 1/2 1/2 1 |  |  |
|  | Sub total | 61/2 |  |  |
| 5 | JOINING THE BACK BODICE TO BACK YOKE WITH MACHINE FELL SEAM (if not give zero) |  |  |  |
|  | - Straight & firm stitchery of the first line - Straight and firm stitchery of the second line close to the fold - Seam allowance neatly tucked under to conceal the raw edges all through the seam - Correct width 6mm to within 2mm and evenness - Flat on the W.S and R.S - Seam pressed downwards | ½ 1 1 1 1 1/2 |  |  |
|  | Sub total | 5 |  |  |
| 6 | MAKING SHOULDER SEAM USING OPEN SEAM (If not give zero) |  |  |  |
|  | - Straight and firm stitchery - Seam pressed open and flat at the neckline and sleeve crown - Seam turnings neatened separately - Correct size of the neatened seam turnings 1cm to within 2mm - Evenness of the neatened seam turnings throughout the length | ½ 1 1 1/2 1 |  |  |
|  | Sub total | 4 |  |  |
| 7 | WORKING SIDE SEAM WITH PLAIN SEAM (if not give zero) |  |  |  |
|  | - Firm and straight stitchery - Seam turnings trimmed to 1 cm and neatened together - Appropriate stitches used (if not give zero from c to f) - Neatening worked with firm and neat stitches. - Correct tension and evenness of the stitches - Stitches fastened on and off - Seam well pressed to the back and flat | ½ 1 ½ 1 1/2 1/2 1 |  |  |
|  | Sub total | 5 |  |  |
| 8 | PREPARATION AND ATTACHMENT OF INTERFACED COLLAR |  |  |  |
|  | - PREPARATION OF THE COLLAR - Collar interfaced - Firm and smoothly curved stitchery - Collar seam trimmed and snipped - Good knife edge - Flatness of the collar | 1/2 1/2 1 1/2 1/2 |  |  |
|  | Sub total | 3 |  |  |
|  | (b)ATTACHMENT OF THE COLLAR USING CROSSWAY STRIP - Cross way strip used (if not give zero from a-d) - Collar and the crossway strip joined to the garment with smoothly and firm stitchery - Seam turnings trimmed and snipped - Crossway strip held in place using appropriate stitches - Collar correctly positioned and flashing at CB to within 2mm - Correct shape and size of the collar | 1/2 1/2 1/2 1/2 1 1 |  |  |
|  | Sub total | 4 |  |  |
| 9 | PREPARATION AND ATTACHMENT OF SLEEVE |  |  |  |
|  | - CONTROLLING FULLNESS AT THE CROWN USING EASING |  |  |  |
|  | - Easing made on the stitching line and 0.6 cm above the stitching line of the sleeve (If not give zero from a-e) - No ruffles made. - Easing stitches not removed. - Easing stitches fastened appropriately on both sides (knots and pin with thread wound to make figure 8) | 1/2 1/2 1/2 1 |  |  |
|  | Sub total | 21/2 |  |  |
|  | - WORKING OF THE FACED SLIT OPENING |  |  |  |
|  | - Three edges of faced slit neatened by edge stitching - Faced slit joined to the sleeve with firm and straight stitches - Slit turned to the WS and well knife edged - The two free edges of the slit held with catch stitches - The opening pressed flat on RS and WS | 11/2 1 1 1 1/2 |  |  |
|  | Sub total | 5 |  |  |
|  | - WORKING OF UNDERARM SEAM (French seam if not give zero) |  |  |  |
|  | - First line of stitches done with firm and straight stitchery. - Second line of stitches done with firm and straight stitchery. - Raw edges completely concealed - Seam well knife edged - Correct size of the finished seam 0.6cm to within 2mm - Evenness of the seam - Seam pressed to the Centre back at the arm hole and at the cuff | 1/2 1/2 1/2 1/2 1/2 1/2 1 |  |  |
|  | Sub total | 4 |  |  |
|  | - PREPARATION AND ATTACHMENT OF CUFF |  |  |  |
|  | - Cuff joined with firm straight stitchery on both sides - Seam turnings trimmed to 0.5cm on both sides. - Both seams well knife edged and the corners sharp - Cuff attached to the sleeve with firm straight stitchery - Seam trimmed to 0.5cm - Cuff finished with hemming stitches (if not give zero from f - k) - Good tension - Evenness of the stitches - Fastened on and off - Firm and neat stitches - Correct size of the cuff - Cuff pressed flat on the RS and WS | 1/2 1/2 1 1/2 1/2 ½ ½ ½ ½ 1 1/2 1 |  |  |
|  | Sub total | 71/2 |  |  |
|  | - ATTACHMENT OF THE SLEEVE |  |  |  |
|  | - Sleeve attached with firm and smoothly curved stitchery - Raw edges and notches matching - Front sleeve joined to the front bodice and vice versa - Seam not trimmed and not neatened - Correct size of the seam allowance and even around the sleeve - Underarm and side seam matching at the armhole to within 2mm - Dot at the crown of the sleeve matching with the shoulder seam - Good hang of the sleeve | 1/2 1/2 1/2 1/2 ½ ½ ½ ½ |  |  |
|  | Sub total | 4 |  |  |
| 10 | HEM MANAGEMENT (machine stitches used if not give zero from b-d) |  |  |  |
|  | - Raw edges completely enclosed by folding 0.5cm and the 2nd fold on the fold line - Machine stitches done if not give zero (b-d) - Correct tension of stitches - Even and neat stitches - Hem flat and even in width - Correct size of the hem | 1 ½ ½ 1 1 ½ |  |  |
|  | Sub total | 41/2 |  |  |
|  | G R A N D T O T A L | 90/2 | X/2 |  |