PAVEMENT FORM 4 TRIAL 1 EXAMINATION 2022

Kenya certificate of secondary education (K.C.S.E)

441/2

HOME SCIENCE

CLOTHING AND TEXTILES

PAPER 2

MARKING SCHEME

| NO. | PROCESES | MAX. SCORE | ACTUAL SCORE | REMARKS |
| --- | --- | --- | --- | --- |
| 1. | Presentation. - Work well pressed (1) and well folded (1). - Label (½) firmly fixed (½) on a single fabric (½) without concealing details.( ½) - Pins (1) and unnecessary tacking (1) removed from work. (1) - Making right half of skirt. (1) | 2 2 3 1 | 8 |  |
|  |  | 8 |  |  |
| 2. | Cutting out. - All five pieces cut out. ( 5x½ ) - Cutting of skirt front, skirt yoke, skirt back, waistband and 2 pocket sections on straight grain. (6x½) - Smooth cutting on CF of skirt front (½) and skirt yoke (½) and CB of skirt back (½). | 2½ 3 1½ | 7 |  |
|  |  | 7 |  |  |
| 3 | Back dart - Straight( ½) stitchery tapering to a point (½) - Dart secured at the point (½) - Correct length of the dart (10cm to within 2 mm. (½) - Correct width of dart.(accurate use of F.L) (½) - Dart pressed towards CB. (½) - Dart flat on R.S (½) and W.S (½) | 1 ½ ½ ½ ½ 1 |  |  |
|  |  | 4 |  |  |
| 4. | Inverted Pleat - Correctly made i.e must be inverted (1) - Correctly held in yoke seam(1) - Good hang (1) - Accurate use of fold lines (2x½) | 1 1 1 1 |  |  |
|  |  | 4 |  |  |
| 5 | Overlaid seam (Yoke seam) - Folding of seam turning of skirt yoke along FL (½) onto the WS (½). - Placing folded edge of yoke on RS (½) of skirt with fold matching FL (½). - Stitch from right side close to fold i.e 2mm from fold (½) - Stitchery straight (½) correct tension (½ ) - Trimming (½) 10cm of seam turnings to 6mm (½) and neatening using loop stiches (1).Loop stitches: even (½) correct size(½) - Flat seam. (1) | 1 1 ½ 1 3 1 |  |  |
|  |  | 7½ |  |  |
| 6. | Inseam pockets. - Correct position (½),size (½) and shape (½). - Reinforced at the mouth i.e understitching of each pocket section. (2x1) - Trimming of seam turnings before under stitching. (2x½) - Inconspicuous on the RS. (1) - Flat (½) - Accurate use of FL when joining pocket sections to make pocket bag. (½) - Unneutered seam turnings of pocket bag.(½) | 1½ 2 1 1 ½ ½ ½ |  |  |
|  |  | 7 |  |  |
| 7 | Side seam: Open neatened seam (Award zero if not open seam) - Accurate use of FL. (1) - Stitchery: correct size (½) correct tension(½) straight (½) - Correct width when finished – 1.2cm for each turning. (2) - Evenness in width (1) - Well neatened (1) - Seam pressed open (1) and flat at waistband (1) and hem (1) on both RS and WS. | 1 1½ 2 1 1 3 |  |  |
|  |  | 9½ |  |  |
| 8. | Waist band - Inner waistband interfaced (½).Interfacing firmly fixed. (½) - Accurate use of FL when attaching waistband.( ½) - Layering of seam turnings. (½) - Correctly positioned waistband i.e flash with CF of front skirt (½) and leave 4 cm overlap at the CB to within 3mm.( ½) - Fixing the waistband on W.S using even (½), correct size(½) of hemming (½)stitches Hemming stitches slanting to the left.( ½) (Award zero if hemming stitches are not used) - Good knife- edging along the edge of the waistband (½) and well-made / sharp corners at the CB (on overlap) (½) - Flatness of the waistband on RS (½) and WS (½) - Evenness of the waistband (½) and correct finished width ( 3 cm) to within 3mm (½) | 1 ½ ½ 1 2 1 1 1 |  |  |
|  |  | 8 |  |  |
|  | TOTAL | 45 |  |  |